ESP32 Board With

16 Channel PWM Board

# GPIO Definition

- Yellow Pin	: Output Signal PWM

- Red Pin		: Output Voltage Supply

- Black Pin		: Ground

Input VCC

*Recommendation +5V

G : Ground

OE : Output Enable

CL : I2C SCL

DA : I2C SDA

IN : Input Logic

V : Input VCC

PWM1 : GPIO 32

PWM2 : GPIO 33

PWM3 : GPIO 25

PWM4 : GPIO 26

PWM5 : GPIO 27

PWM6 : GPIO 14

PWM7 : GPIO 12

PWM8 : GPIO 13

PWM9 : GPIO 2

PWM10 : GPIO 4

PWM11 : GPIO 16

PWM12 : GPIO 17

PWM13 : GPIO 5

PWM14 : GPIO 18

PWM15 : GPIO 19

PWM16 : GPIO 23

# Spesification

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VCC | Supply Voltage for Output Pin | - | 5 | 5.3 | V |
| IN | Supply Voltage For Logic (Microcontroller) | 3.3 | - | 5.3 | V |
| VIH | High Level Input Voltage | - | - | 3.3 | V |
| VIL | Low Level Input Voltage | 0 | - | - | V |
| VOH | High Level Output Voltage | - | 3.3 | - | V |
| VOL | Low Level Output Voltage | 0 | - | - | V |
| Freq | Output Frequency Signal | - | - | 40 | Mhz |

# Information

In the red box in the picture there is a jumper pad. connected if the VCC Input also activates the logic supply (microcontroller). if not connected then the supply will be connected independently.

# Example Code

#define PWM_1 32

#define PWM_2 33

#define FREQUENCY 5000

#define BIT_WIDTH 12

void setup(){

Serial.begin(115200);

ledcAttach(PWM_1, FREQUENCY, BIT_WIDTH);

ledcAttach(PWM_2, FREQUENCY, BIT_WIDTH);

}

void loop(){

ledcFade(PWM_1, 0, 4095, 2000);

ledcFade(PWM_2, 0, 4095, 2000);

delay(1000);

}